OUT OF RANGE

Unit CGMU4941980 Model: LXE10E-A15J and with Set Temp of -21.0C.

1. Thanks for the efforts in repairing the unit.

2. Based on RA -16.1C and if ambient temp is +28.0C to +33.0C; unit discharge manifold pressure or HPT display should be 1250Kpa to 1300Kpa.

2a. Based on RA -16.1C and if ambient temp is +34.0C to +40.0C; unit discharge manifold pressure or HPT display should be 1450Kpa to 1500Kpa.

3. Before gas release or top up, do assist to compare discharge manifold pressure with HPT displays. Are both discharge pressure the same? If different, possibly HPT PN: 158795J defective.

4. Do ensure EIS temp readings is cooler than EOS temp, if not compressor body will heavily frost-up and compressor body temp or DCHS display should be +70C to +98.0C. If these 2readings are NOT match, do provide us parameter values.

5. To ensure unit in good performance; do provide details value on display -- RA, SA, DRA, DSA, HPT, LPT, CT1, CT2, AMB, DCHS, EIS, EOS, MV, SGS and EV.

6. Do assist to reset defrost timer 99hourly for any frozen setting; unit is programmed will auto defrost every 4hourly if unit is out of range.

Unit MORU0709694 Model: LXE10E-A37 and with Set Temp of +4.0C.

1. Thanks for the MAL report.

2. Do assist to compare discharge manifold pressure with HPT displays. Are both discharge pressure the same? If different, possibly HPT PN: 158795J defective.

3. Base on Compressor body temp is 110.2C. Do carry-out Short PTI from P01 to P08 test. After completion P08 Pump down test, do power OFF and ON. These is to initial EEV opening.

4.Do assist to Clean and service EEV body & coil orifice, follow by pushing a few times on the spring orifice on EEV body to remove water condensation or residue at the EEV body bellow. Refit EEV coil. Do note, water in EEV bellow will cause freeze up during operation, causing EEV operates irregular.

5. Do observe EIS and EOS temp reading again. In case, EIS temp reading -16.6C is warmer than EOS temp -20.1C; do temporary isolate EIS wires, alarm E407 will be activated but just ignore alarm.

6.  In case, box temp not improving or alarms activation; do provide details value on display -- RA, SA, DRA, DSA, HPT, LPT, CT1, CT2, AMB, DCHS, EIS, EOS, SMV, SGS and EEV.

HOT CARGO

Unit FSCU5689463 Model: LXE10E136B4R and with Set Temp of -4.0C.
1. Thanks for the MAL report.
2. Base on RA and SA Temp values was beyond 5 degree C difference which mean unit is hot cargo inside box.
3. Do assist to check 2pcs EF motor air blow should be downward it will blow the EF coil.
4. Do assist to observe drain hose, is there any water comes out from drain hose during and after defrosting?
5. Please do check is drier outlet piping cooler than inlet piping or sweating on drier filter & piping? If yes, replace drier (suspect choked).
6. Once, unit in operation; do provide details value on display -- RA, SA, DRA, DSA, HPT, LPT, AMB, DCHS, CT1, CT2, EIS, EOS, SMV, SGS and EEV for our reference.

DEF. DISPLAY/BORROW

Unit BMOU9715429 Model: LXE10E133A1 and with Set Temp of -18C,

1. Thanks for feedback.

2. Since no spare Display module on board and no empty reefer to cannibalize.

3. Do assist to temporary barrow Display module from other unit, to check the RA and SA temp during monitoring time.

4. We will arrange shore assistance at next port.

EEV CHOKED

Gdays Master and Chief Engineer of George Washington Bridge,

Unit NYKU7157027 Model: LXE10E-A26 with Set Temp of -25.0C.

1.As LPT reads -47 means unit is running under vacuum state.

2. Do advise the EV opening  whether the opening is above or below 10%.

3. If EEV opening percentage is less than 10%, high chances EEV coil jammed / EV body choked.

4. Kindly reconnect EIS sensor and restart the unit and perform Short PTI till SP32 and revert if any alarm triggered .

5. Make sure there is no any service valve leaking, since it might cause unit running under vacuum as well.

6. Revert us back with the parameter -- RA, SA, DRA, DSA, HPT, LPT, CT1, CT2, AMB, DCHS, EIS, EOS, MV, SGS and EV.

Gdays CMA CGM MANZANILLO, MX, MX Team,

Do assist to arrange shore assistance for unit TCLU1033943 upon vessel arrival. Vessel ETA:13/11/2019.

Dear Master of Cosco Shipping Jasmine,

Unit : CGMU4935566 : Model : LXE10E-A15J Setpoint : -18c

1.Thanks for the malfunction report.

2. Alarm F803 may activate with others alarms 101, 107, 109 which activated more than 3 times.

3. Check whether Phase or Compressor contactors in poor contact or Contactor low resistance or Contactor jammed or comp not operating. Do assist to check terminal Fuse #1, is 10amp fuse blown?

4. Check Both EF Motors whether does it stop abnormally or current drawn abnormal.

5. Kindly check for discharge operating pressure, high / low pressure transducer out of range or system gas shortage due to leakage.

6. Based on RA -17.5C and ambient temp is 33.5C, discharge pressure on manifold gauge should be around 1050KPA TO 1100KPA. Top up ig gas lvl low.

6. Please do check is drier outlet piping cooler than inlet piping or sweating on drier filter & piping? If yes, replace drier (suspect choked).

7.Do assist to observe is EEV assembly being covered with frost ice, EEV coil flange loosen of EEV coil shaft jammed.

8. If yes, do assist to Clean and service EEV body & coil orifice, follow by pushing a few times on the spring orifice on EEV body to remove water condensation or residue at the EEV body bellow. Refit EEV coil. Do note, water in EEV bellow will cause freeze up during operation, causing EEV operates irregular.

9. Do assist to trace gas leakage or oil stained. Gas leakage can be from servicing core valves worn out causing leakage or drier inlet / outlet flaring worn out or loose connection or other HPT / LPT capillary tubing or joint leaking.

10. Please check on sensors readings, sometimes if any sensors shorted, alarm F803 may appear.

11. Do observe display “PT” showing 380 - 440volts, “CT1” displaying total current and “CT2” displaying compressor current.  If one of the reading abnormal or fluctuating or reflecting actual current draws of the component, replace PTCT board 12. Replace controller if above found to be OK.

13. Do provide us the detailed parameters of RA, SA, DRA, DSA, HPT, LPT, PT, CT1, CT2, AMB, DCHS, EIS, EOS, SMV, SGS and EV.

Container ID: KKFU6964070, Model: LXE10E-A9C & SP: -18C.

1. thanks for effort and feedback.

2. In case, alarm F803 is associate with E109 and E107; do follow below instructions.

3. Do assist to observe is EEV assembly being covered with frost and ice or EEV coil flange loosen.

4. If yes, do assist to Clean and service EEV body & coil orifice, follow by pushing a few times on the spring orifice on EEV body to remove water condensation or residue at the EEV body bellow. Refit EEV coil. Do note, water in EEV bellow will cause freeze up during operation, causing EEV operates irregular.

5. Observe again, is LPT displays or suction manifold pressure should have pressure now? Do ensure system have suffice gas.

6. If gas level is normal, follow up next. Do assist to observe both EIS and EOS temp readings; if EIS temp is warmer than EOS temp. Please assist to OFF the unit 3 to 5 minutes and re-ON it.

7. Observe again, EIS and EOS temp readings. If EIS temp still warmer than EOS, temporary isolates EIS wires, alarm 407 will activate, just ignore and not affecting unit performance.

8. To ensure unit operates in good condition, please provide details value on display -- RA, SA, DRA, DSA, HPT, LPT, CT1, CT2, AMB, DCHS, EIS, EOS, MV, SGS and EV.

9. Do assist to reset defrost timer 99hourly for any frozen setting; unit is programmed will auto defrost every 4hourly if unit is out of range.

Unit CGMU4955834 Model: LXE10E-A15J and with Set Temp of -18.0C.

1. Thanks for the efforts in repairing the unit.

2. Based on RA -16.4C and if ambient temp is +32.2C; unit discharge manifold pressure or HPT display should be 1100Kpa to 1150Kpa. Do top up gas.

3. Before gas release or top up, do assist to compare discharge manifold pressure with HPT displays. Are both discharge pressure the same? If different, possibly HPT PN: 158795J defective.

4. Do ensure EIS temp readings is cooler than EOS temp, if not compressor body will heavily frost-up and compressor body temp or DCHS display should be +70C to +98.0C. If these 2readings are NOT match, do provide us parameter values.

5.  In case, box temp not improving or alarms activation; do provide details value on display -- RA, SA, DRA, DSA, HPT, LPT, CT1, CT2, AMB, DCHS, EIS, EOS, MV, SGS and EV.

6. Do assist to reset defrost timer 99hourly for any frozen setting; unit is programmed will auto defrost every 4hourly if unit is out of range.

7. In case, HPT readings is different from Discharge manifold gauge pressure and No spare HPT on board. Do just disconnect HPT wires.

Container ID: CGMU5447859 Model: LXE10E-36JA & SP: -3.0C.

1.Thanks for the detail values as we requested.

2. I noticed the High Pressure reading very high: 2150kpa, Discharge gas temp also high 118C; HPT increase due to ambient air is very high.

3.You must check the condenser coil and wash it if there any dirt.

4. Please do place additional blower to cool down the ambient air.

5. Please do check the condenser fan motor if it is rotate or function correctly. Input and output voltage of CF contactor must also be check.

6. Due to high ambient and high pressure, may encounter frequent high cut  by HPS. May temporary bypass HPS wires; but ensure discharge operating pressure NO more 2500Kpa. If more than 2500Kpa do release some gas.

7. Do reconnect HPS wires, when discharge operating pressure less than 1800Kpa or unit to be discharging soon.

8. May temporary close the Air vent and re-open to actual setting when box temp reached within range or when unit to be discharging soon.

9. Do temporary isolate EIS wires and alarm E407 will be activated but just ignore alarm.

ALARM E109

Unit CGMU4947968 Model:LXE10E-A15J with setpoint -21C,

1. Thanks for the efforts in repairing the unit. If unit alarm appears again, kindly follow below instruction.

2. Alarm E109 is system gas shortage due to leakage or system under vacuum. Do assist to observe is EEV assembly being covered with frost ice, EEV coil flange loosen of EEV coil shaft jammed.

3. If yes, do assist to Clean and service EEV body & coil orifice, follow by pushing a few times on the spring orifice on EEV body to remove water condensation or residue at the EEV body bellow. Refit EEV coil. Do note, water in EEV bellow will cause freeze up during operation, causing EEV operates irregular.

4. Do assist to top up gas if unit gas level is too low.

5. Do assist to trace gas leakage or oil stained. Gas leakage can be from servicing core valves worn out causing leakage or drier inlet / outlet flaring worn out or loose connection or other HPT / LPT capillary tubing or joint leaking.

6. Please do check is drier outlet piping cooler than inlet piping or sweating on drier filter & piping? If yes, replace drier (suspect choked).

7. Please provide details value on display -- RA, SA, DRA, DSA, HPT, LPT, CT1, CT2, AMB, DCHS, EIS, EOS, MV, SGS and EV.

Good days Master of  SHANGHAI TRADER,

Unit: CGMU4936032  Model: LXE10E A15J Set temp.-20.0C.

1.Thanks for the feedback. Suspect cargo wet load, box temp will improve gradually.

2.Based on the RA Temp -10.5C and if ambient temp is +28.0C to +33.0C; unit discharge manifold pressure or high discharge display should be 1500kpa to 1550kpa. Do top up gas if discharge pressure low.

3. Before gas release or top up, do assist to compare discharge manifold pressure with HPT displays. Are both discharge pressure the same? If different, possibly HPT PN: 158795J defective.

4. Do ensure EIS temp readings is cooler than EOS temp, if not compressor body will heavily frost-up and compressor body temp or DCHS display should be +70C to +98.0C. If these 2readings are NOT match, do provide us parameter values.

5. To ensure unit in good performance; do provide details value on display -- RA, SA, DRA, DSA, HPT, LPT, CT1, CT2, AMB, DCHS, EIS, EOS, MV, SGS and EV.

6. Do assist to reset defrost timer 99hourly for any frozen setting; unit is programmed will auto defrost every 4hourly if unit is out of range.

BYPASS REPAIR COTROLLER DECOS III D AND III C

Unit CGMU4938333 Model: LXE10E-A15J and Set temp -2.0C.

1. Thanks for the feedback.

2. Do assist to observe is SMV working? On the front top PC board is MV board, do check is MV board blown OFF. Is HOT gas Solenoid coil and valve working?

3. On front Controller, there SMV switches, in case SMV not working please make the dip switches are on the position as shown in picture below

4. Emergency has not control on pre-set temperature may require to OFF and ON to maintain box temp and avoid overcool on chill mode.

BYPASS REPAIR COTROLLER DECOS III D AND III C

Unit CGMU4938333 Model: LXE10E-A15J and Set temp -2.0C.

1. Thanks for the feedback.

2. Do assist to observe is SMV working? On the front top PC board is MV board, do check is MV board blown OFF. Is HOT gas Solenoid coil and valve working?

3. On front Controller, there SMV switches, in case SMV not working please make the dip switches are on the position as shown in picture below

4. Emergency has not control on pre-set temperature may require to OFF and ON to maintain box temp and avoid overcool on chill mode.

EEV emergency cap.. Emergency mode

Good day Master and Chief Engineer of THALASSA HELLAS ,

BYPASS REPAIR COTROLLER DECOS III F

Unit SZLU9638529 Model: LXE10E-136F2 and with set temp of -20.0C.

1. Thanks for MAL report.

2. Kindly put back the original connection and follow below instructions.

3. Do restart unit when display still indication green but compressor not running. do measure output voltage from I/O board CN19 terminal 1 & 2. value to measured is 24V.

4. If have 24V output, do turn OFF unit and measure compressor coil (CC) resistance A1 & A2. Value to be measured 3.7Ω. If low resistance. do replace.

5. If resistance is OK, do try to replace RPP.

6. If above procedure been taken but still problem remain. do find attached controller emergency operation.

BYPASS REPAIR COTROLLER DECOS III E

Unit BMOU9732072 Model: LXE10E136A19 and with set temp -5.0C.

1. Thanks for the efforts and  feedback.

2. Do assist to perform by-pass operation on unit for temporary repair to let temperature pull down to required temp and read box temperature by placing thermometer.

3. Please find attached the procedure of emergency by-pass operation.

4. Since unit is in chilled mode setting the unit power should be cycle using OFF/ON switch in order to prevent the unit over cooling.

5. Please revert the result.

Unit TGHU9918484 Model LXE10E-A36A with set temp -20.0C.

1. Thanks for the efforts in repairing the unit.

2. If you decide to use back EEV coil, do assist to Clean and service EEV body & coil orifice, follow by pushing a few times on the spring orifice on EEV body to remove water condensation or residue at the EEV body bellow. Refit EEV coil. Do note, water in EEV bellow will cause freeze up during operation, causing EEV operates irregular.

3. Based on RA -19.8C and if ambient temp is +25.0C; unit discharge manifold pressure or HPT display should be 1000Kpa to 1050Kpa.

4. Before gas release or top up, do assist to compare discharge manifold pressure with HPT displays. Are both discharge pressure the same? If different, possibly HPT PN: 158795J defective.

5. Do ensure EIS temp readings is cooler than EOS temp, if not compressor body will heavily frost-up and compressor body temp or DCHS display should be +70C to +98.0C. If these 2readings are NOT match, do provide us parameter values.

6.  In case, box temp not improving or alarms activation; do provide details value on display -- RA, SA, DRA, DSA, HPT, LPT, CT1, CT2, AMB, DCHS, EIS, EOS, MV, SGS and EV.

7. Do assist to reset defrost timer 99hourly for any frozen setting; unit is programmed will auto defrost every 4hourly if unit is out of range.

8. In case, prefer unit continue using EEV emergency cap; do assist to remove before unit discharging for repairing port.

SHORE & UNDERCHARGE

Unit : SEGU9303809 Model : LXE10E-136A Setpoint : -25c

1. Thanks for the malfunction report.

2. As per current parameters, unit running normal except with low freon level.

3. Do check whether any cool air leakage on air vent panel. If yes, kindly open EF panel and check for any ice formation. 4. Clear if any ice formation occurred. Temporarily tape up air vent panel with aluminium tape and tighten wingnut.

Dear One.Shipspare Team,

Do kindly assist to replenish r134a as vessel don’t have refrigerant anymore

EEV PROBLEM

Unit MORU0612030 Model:LXE10E-A12C,

1.Thank for the malfunction report .

2. Do assist to observe is EEV assembly being covered with frost ice, EEV coil flange loosen of EEV coil shaft jammed.

3. If yes, do assist to Clean and service EEV body & coil orifice, follow by pushing a few times on the spring orifice on EEV body to remove water condensation or residue at the EEV body bellow. Refit EEV coil. Do note, water in EEV bellow will cause freeze up during operation, causing EEV operates irregular.

4. After step 3 and compressor still ice up , do assist to perform SPTI 01 to SPTU 10 .

5. After SPTI test , Reset the power and RUN the unit . compressor ice again ? do assist to eev coil (2238128) .

6. Please do check is drier outlet piping cooler than inlet piping or sweating on drier filter & piping? If yes, replace drier (suspect choked).

7. Please provide details value on display -- RA, SA, DRA, DSA, HPT, LPT, CT1, CT2, AMB, DCHS, EIS, EOS, MV, SGS and EV.

EEV III E

Container ID: TGHU9942449, Model: LXE10E-136A1 & SP: -18C.

1. thanks for effort and feedback.

2. Do assist to check gas level normal?

3. Do assist to check EEV coil is secure properly and resistance of EEV Coil. If resistance low, replace it. PN: 1890819 or 2266851.

4. Do clean & Clear EEV assembly frost if being covered with frost or ice.

5. Digital EEV coil 46 +/-3 OHM [White - Red: 46Ω, Orange - Red: 46Ω, Yellow - Brown: 46Ω and Blue - Brown: 46Ω].

6. Please do check is drier outlet piping cooler than inlet piping or sweating on drier filter & piping? Any oil stained or gas leakage at inlet/outlet drier flaring?  If yes, replace drier.

7. Do assist to check LPT reading with suction manifold pressure, is it the same? If not, do check on wiring/sockets at terminal board and re-secure if loose.

8. Please assist to provide details value on display -- RA, SA, DRA, DSA, HPT, LPT, CT1, CT2, AMB, DCHS, EIS, EOS, MV, SGS and EV.

Unit MORU0603064 Model: LXE10EA12C and with Set Temp of -18.0C.

1. Thanks for the efforts in repairing the unit. Temporary install back EEV coil, there is nothing wrong. Suspect system gas shortage.

2. Based on RA -15.2C and if ambient temp is +29.8C; unit discharge manifold pressure or HPT display should be 1150Kpa to 1200Kpa.

3. Before gas release or top up, do assist to compare discharge manifold pressure with HPT displays. Are both discharge pressure the same? If different, possibly HPT PN: 158795J defective.

4. Do ensure EIS temp readings is cooler than EOS temp, if not compressor body will heavily frost-up and compressor body temp or DCHS display should be +70C to +98.0C. If these 2readings are NOT match, do provide us parameter values.

5.  In case, box temp not improving or alarms activation; do provide details value on display -- RA, SA, DRA, DSA, HPT, LPT, CT1, CT2, AMB, DCHS, EIS, EOS, MV, SGS and EV.

6. Do assist to reset defrost timer 99hourly for any frozen setting; unit is programmed will auto defrost every 4hourly if unit is out of range.

AL F101 PHASE PROBLEM

1. Thanks for provide the malfunction report.

2. Do assist to measure voltage at input / output of main breaker. Possible voltage abnormal. Do assist to counter check, any loose wire connection or 440v plug partial burnt? Do counter on splicing kit joint on 440v power cable, possibly poor contact or open circuit.

3. Do assist to advise, is anyone of phase contactor energies? If No, do assist to measure the resistance of both phase contactors and compressor contactor; is there any low resistance? If Yes, replace Contactor PN: 1198917 or 119891J. Compressor / phase contactor --- 3.7Ω    124149J or 1241493 phase contactor. PN: 119891J or 1198917 compressor contactor.

4. Please observe display "PT" showing 380 - 440volts, "CT1" displaying total current and "CT2" displaying compressor current.  If one of the reading abnormal or fluctuating or reflecting actual current draws of the component, replace PTCT board PN: 0991050 or PN: 2139904.

5. Please provide details value on display -- RA, SA, DRA, DSA, HPT, LPT, CT1, CT2, AMB, DCHS, EIS, EOS, MV, SGS and EV.

Unit CGMU6543440 Model: LXE10E-136HC and with Set Temp of -18.0C.

1. Thanks for MAL report.

2. Do assist to measure voltage at input / output of main breaker. Possible voltage abnormal. Do assist to counter check, any loose wire connection or 440v plug partial burnt? Do counter on splicing kit joint on 440v power cable, possibly poor contact or open circuit.
3. Do assist to advise, is anyone of phase contactor energies? If No, do assist to measure the resistance of both phase contactors and compressor contactor; is there any low resistance/stuck-up? If Yes, replace Contactor PN: 1198917 or 119891J. Compressor / phase contactor --- 3.7Ω    124149J or 1241493 phase contactor. PN: 119891J or 1198917 compressor contactor.

4. If no spare, do assist cannibalize from any empty CMA/APL reefer unit. do provide container number and POD/ETA if part was cannibalized.

5. Please observe display "PT" showing 380 - 440volts, "CT1" displaying total current and "CT2" displaying compressor current.  If one of the reading abnormal or fluctuating or reflecting actual current draws of the component, replace PTCT board PN: 0991050 or PN: 2139904.

6. Do revert outcome and please provide details value on display -- RA, SA, DRA, DSA, HPT, LPT, CT1, CT2, AMB, DCHS, EIS, EOS, MV, SGS and EV.

Unit KKFU6957528 Model: LXE10EA9 and with Set Temp of -20.0C.

1. Thanks for effort in trouble shoot the Unit.

2. Before replacing board have you do any cutting of Diode as per attachments.

3. Do assist to advise, is anyone of phase contactor energies?

4. If unit switch is On, Phase PCC1, any 24Vac Voltage at wire 143 and 104/100? If yes, Any Voltage at wire153 and 104/100? If No. any  poor contact between 143 and 153?

5. If unit switch is On, Phase PCC2, any 24Vac Voltage at wire 145 and 104/100? If yes, Any Voltage at wire154 and 104/100? If NO, any poor contact between 145 and 154.

6. If one of Phase Contactor is energies and compressor contactor NOT energies. Do assist to measure any 24Vac at wire 133 of compressor contactor (CC) and wire 100/ 104?  If NO, Do check RPP or Jumper 133 and 155. If yes, do check Wire 155 (CC) and wire 100 / 104 (negative wire) any 24vac. If no, possibly HPS open circuit. May temporary bypass HPS wires 130 and 131.

7. Do assist to bypass HPS wires (129 and 130); do compressor contactor energies?

8. Compressor / phase contactor --- 3.7Ω. replace contactor is coil resistance low.

9. Do counter check Fuse #1, #3 and #7 on I/O board for any blown? Replace 10A fuse, if blown.

10. Do place a jumper on wire 101 to 133 terminal, if after HPS bypassed compressor contactor cannot energies.

11. May operate unit under emergency mode. But, care must be taken due to NO control pre-set temp when operating on Emergency mode to power OFF and ON.

EEV BODY BYPASS

Unit KKFU6952193 Model LXE10E-A9B and with Set Temp -18.0C,

1. Thanks for feedback.

2. Do assist to run the unit without EEV coil. Unit still vacuum suction or have Alarm E109?

3. If unit still Vacuum suction possible problem is Clogged EEV body. Replace EEV body

4. If don't have spare on board, Do aasist to bypass the EEV body as per attached image. Use hand valve to control the pressure.

5. Do revert the outcome to us.

6. We will arrange Shore assistance at next port TOKYO ,JAPAN. Vessel ETA 03rd January.

LSV / EEV

Unit NYKU7131450 Model: LXE10E-A26C with set temp of -18.0C.

1. Thanks for the efforts in repairing the unit.

2. Do assist to compare the LPT/HPT display with accurate manifold gauge, if value different, try to replace LPT 1587942 or 2398734, HPT 1587959 or 2289948.

3. Do find attached for another location of service port which is located before the filter drier.

4. Do assist to remove EEV coil and run the unit and observe suction manifold pressure and LPT display. Still vacuum?

5. After performing Liquid line Strainer and LSV by-pass, still vacuum operation, suspect EEV body choked, do replace if spare on board or if brazing is possible on board. Do extra care when replacing EEV body, wrap it with wet rag during brazing to avoid overheating. EEV overheated can cause EEV gas leakage.

6. Do provide manifold running pressure of all service ports, do find attached photo for service port location and numbering.

7. Do revert outcome.

LSV

Unit BMOU9706788 Model: LXE10E-A15J and with Set Temp of -25.0C.
1. Thanks for the efforts and feedback.
2. With unit compressor in operation, do assist to provide pressure readings for Servicing Ports, attached are the location of service ports: Do ensure that the compressor is running while taking the value on each service port.
1.) Suction Service Port.
2.) Discharge Service Port.
3.) Liquid Line Service Port.
4.) Discharge Pressure after Check Valve.
5.) Discharge Pressure after Receiver or Water Cool Condenser.
3. Do counter check LSV coil, do measure LSV coil resistance, value to be measured 15.2 ohms to 16.0 ohms and voltage during compressor operation is 24 volts at Terminal board connector 14. Do assist to check for any loose or poor connection of LSV coil.
4. Do assist to check whether Liquid line strainer (after receiver tank) is choked, may by-pass Liquid line strainer if it confirms choked as photo attached.
5. Please revert with findings.

HIGH TEMPERATURE

Unit SZLU9624340 Model: LXE10E136B2 and with Set Temp of +6.0C.

1. Thanks for the efforts in repairing the unit.

2. Based on RA +12.9C and if ambient temp is +27.0C; unit discharge manifold pressure or HPT display should be 1050kpa to 1100Kpa.

2a. Based on RA +12.9C and if ambient temp is +28.0C to +33.0C; unit discharge manifold pressure or HPT display should be 1250kpa to 1300Kpa.

3. Before gas release or top up, do assist to compare discharge manifold pressure with HPT displays. Are both discharge pressure the same? If different, possibly HPT PN: 158795J defective.

4. Do ensure EIS temp readings is cooler than EOS temp, if not compressor body will heavily frost-up and compressor body temp or DCHS display should be +70C to +98.0C. If these 2readings are NOT match, do provide us parameter values.

5. To ensure unit in good performance; do provide details value on display -- RA, SA, DRA, DSA, HPT, LPT, CT1, CT2, AMB, DCHS, EIS, EOS, MV, SGS and EV.

6. AlarmF803 may activate with others alarms 101, 107, 109 which activated more than 3 times.

7.  Phase or Compressor contactors in poor contact or Contactor low resistance or Contactor jammed or comp not operating. Do assist to check terminal Fuse #1, is 10amp fuse blown?

8. Both EF Motors stop abnormal or current drawn abnormal.

9. High discharge operating pressure, high / low pressure transducer out of range or system gas shortage due to leakage.

10. If any Two of HPT, LPT and DCHS failed.

11. Associate with Fd 01 to 05 alarms.

12. Do observe display “PT” showing 380 - 440volts, “CT1” displaying total current and “CT2” displaying compressor current.  If one of the reading abnormal or fluctuating or reflecting actual current draws of the component, replace PTCT board PN: 0991050 or PN: 2139904 or PN: 2289690.

13. Replace controller if above found to be OK.

14. Enclosed Daikin bulletin F803 for your reference.

FLACTUATE SENSOR READING

Unit: CAIU5415655 Model:LXE1OE-A36A with set temp of +12.0C

1.thanks for the Mal report.

2.As i observed RA temp reading is irregular and fluctuating. Is unit on defrosting when RA temp high?

3.IF NO, do temporary use Ambient sensor to replace Return sensor.

4. Before replacing of RA sensor, do ensure both EF Fan rotating in correct rotation.

2. As i observed RA temp reading is irregular and fluctuating. Is unit on defrosting when RA temp high? Do advise is recorder return sensor temp. is the same as control return air temp.

3.IF NO, do replace Return sensor.4. Before replacing of RA sensor, do ensure both EF Fan rotating in correct Rotation.

AL F807.COMPRESSOR PROBLEM

Container ID: CGMU5442456, Model: LXE10E136JA & SP: -3C.

1. Thanks for the efforts in repairing the unit.

2. F807 means Compressor connection disconnected.

3. Do assist to measure voltage at input / output of main breaker. Possible voltage abnormal. Do assist to counter check, any loose wire connection or 440v plug partial burnt? Do counter on splicing kit joint on 440v power cable, possibly poor contact or open circuit.

4. Do switch to another good power point, possibly internal receptacle wire burnt.

5. Do assist to advise, is anyone of phase contactor energies? If No, do assist to measure the resistance of both phase contactors and compressor contactor; is there any low resistance? If Yes, replace Contactor PN: 1198917 or 119891J. Compressor / phase contactor --- 3.7Ω    124149J or 1241493 phase contactor. PN: 119891J or 1198917 compressor contactor.

6. Please observe display “PT” showing 380 - 440volts, “CT1” displaying total current and “CT2” displaying compressor current.  If one of the reading abnormal or fluctuating or reflecting actual current draws of the component, replace PTCT board PN: 0991050 or PN: 2139904.

7. If one of Phase Contactor is energies and compressor contactor NOT energies. Do assist to measure any 24Vac at wire 133 of compressor contactor (CC) and wire 100/ 104? Wire 155 (CC) and wire 100 / 104 (negative wire). May temporary bypass HPS wires, if No voltage input. Release gas if discharge pressure higher than 2200Kpa, because no protection on compressor as HPS wires being bypassed.

8. Do counter check on compressor junction box, is there any terminal wires burnt or loose connection?

9. Once, unit in operation; do provide details value on display -- RA, SA, DRA, DSA, HPT, LPT, AMB, DCHS, CT1, CT2, EIS, EOS, MV, SGS and EV for our reference.

F807

Container ID: BMOU9867078 Model: LXE10E136J2 & SP: -23.0C

1. Thanks for the efforts and support.

2. If same alarm comes back. Do assist to replace the I/O board 2266844. Suspect I/O board miniature relay jammed.

3. If no spare on board, cannibalize from empty or dry reefer under ONE fleet on board and provide cannibalized unit number and POD if part been cannibalized.

ALE205

Unit BMOU9735261 Model LXE10E-136A1 and with set temp +12.0C

1. Thanks for provide the malfunction report. Do assist to check power plug / plug pin / vessel power point .

2. For alarm E205, please take EF motor amperes reading using ampere meter and compare it with ampere reading prompted on display using manual function test on EF motors high / low operation or fan blade stuck by ice block.

3. There are 2wires each on terminal of EF contactor, measure current draw on each wire on side of 3terminal; which represent motor left and right. Measure ampere of each wire instead of two wires together, sometimes one motor overheat may affect the reading when both wires taken together.

4. Please check any poor contact on EF contactor or fluctuating of motor current drawn cause by PTCT board PN: 0991050 0r 2139904 or 2289690.

5. May try by changing 2 phase wires on the 440v plug, that mean L1 & L2 or L2 & L3 interchange on 440v plug and sometimes it worked.

Unit NYKU7174425 Model: LXE10E-A26E and with Set Temp of -20.0C.
1. Thanks for the MAL report.
2. Since EF motor #1 was defective, do assist to isolate/disconnect EF motor #1 wires on contactor and then operate the unit with EF motor #2 only.
3. On the faulty EF motor bell mouth access, kindly place a card board to avoid cool air short cycling.
4. Kindly revert outcome.

Unit CGMU4938040 Model:  LXE10E-A15J and with set temp of 1.0C.

1. Thanks for MAL report.

2. Do continue to disconnect faulty motor & operate the unit with one EF motor only.

3. On the faulty EF motor bell mouth access, kindly place a card board to avoid cool air short cycling.

4. As unit can run in-range, do assist to continue monitor till POD & just ignore alarm E205.

5. We will arrange shore assistance at DAMIETTA; repair will follow-up when unit discharge to yard.

Gdays CMA CGM DAMIETTA, EG Team,

Terminal: DAMIETTA CONTAINER CARGO HANDELIN

Do assist to arrange shore assistance for unit CGMU4938040, upon unit discharge to yard. Vessel ETA: 13th December.

NOTE: "Need to replace EF motor no. 2"

ALE107

1. Thanks for the feedback.

2. Alarm E107 is high discharge compressor temp and might due to HPT out of range, drier choked, system gas shortage, mixture of air in the system or compressor sensor defective or choked at economizer receiver strainer or liquid solenoid valve restriction or its solenoid coil defective. Any repeated alarms for more than 3 times, F803 will also activated.

3. Please assist to fit a manifold gauge on discharge svc port and compare it with HPT display. If both pressures are difference, replace High pressure transducer or Schrader valve pin poor contact causing restriction gas flow.

4. Place your finger on each port valve and release, is there any gas comes out? If yes, valve core defective . Do replace valve core, fully vacuum system and top up r134a accordingly to required kg, stated in container’s nameplate.

5. Based on RA -8c with ambient temp 20c, discharge pressure on manifold gauge should be around 900kpa to 950kpa.

6. Please remove EEV coil & observe EEV body’s orifice for any ice freeze up. Clean and refit EEV coil. If suction pressure failed to rise after cleaning, replace EEV Coil.

7. To ensure unit operates in good condition, please provide details value on display -- RA, SA, DRA, DSA, HPT, LPT, PT, CT1, CT2, AMB, DCHS, EIS, EOS, MV, SGS and EV.

SLOW PULL DOWN

Unit NYKU7152278 Model:LXE10E-A26E with setpoint -20C,

1.Thank for the malfunction report .

2.Base on RA -15.8C with ambient +14.6C , unit discharge pressure should be 700kpa to 800 kpa . Do assist to remove gas slowly .

3.After remove gas , do assist to observe compressor temperature still high ? if yes , do assist to vacuum system and top up gas as name plate

MOISTURIZE

1.Thanks for the repairs carried out.

2. Based on RA -19.2c with ambient temp 39c, discharge pressure on manifold gauge should be around 1200kpa to 1250kpa. Do assist to top up / release gas accordingly.

3. If compressor temp increases, suspect air mixture in system. Do assist to vacuum system and top up r134a as required kg, stated in container's nameplate.

3. Do monitor the unit closely and revert if any abnormalities arise.

Unit SEGU9322352 Model: LXE10E-136 and with Set Temp of -18.0C.

1. Thanks for efforts & feedback.

2. Do assist to compare discharge manifold pressure with HPT displays. Are both discharge pressure the same? If different, possibly HPT PN: 158795J defective.

3. Do try to bypass/jumper HPS & observe unit.

4. If temperature still not improving but unit can run, suspect air mixture in system. Do assist to replace filter drier & perform deep vacuum on system and top up fresh r134a gas as required kg, stated in container's nameplate.

5. In case, box temp not improving or any other alarms exist; do provide details value on display -- RA, SA, DRA, DSA, HPT, LPT, CT1, CT2, AMB, DCHS, EIS, EOS, MV, SGS and EV. Also provide suction & discharge manifold pressure.

COMPRESSOR ADVICE

Unit : SZLU9714300 Model : LXE10F11B Setpoint : 18c

1. Noted on compressor defective.

2. Kindly send us the available compressor image attached with part no.

3. If no spare on board, kindly cannibalise from any empty One.Line LX10F reefer on board. IF part been cannibalised, do provide us the container no and POD.

ALJ291

Unit : NYKU7173100 Model : LXE10E-A26E Setpoint : -22c

1. Thanks for the efforts in repairing the unit.

2. Based on RA -19.8c and if ambient temp is 18.0C; unit discharge manifold

pressure or HPT display should be 800kpa to 850kpa.

3. Before gas release or top up, do assist to compare discharge manifold

pressure with HPT displays. Are both discharge pressure the same? If different,

possibly HPT PN: 158795J defective.

4. Do ensure EIS temp readings is cooler than EOS temp, if not compressor body

will heavily frost-up and compressor body temp or DCHS display should be +70C

to +98.0C. If these 2readings are NOT match, do provide us parameter values.

5. Do assist to reset defrost timer 99hourly for any frozen setting; unit is

programmed will auto defrost every 4hourly if unit is out of range.

6. Two reasons for compressor body heavily frost up can be due to system gas

overcharged or EIS temp reading out of range. Refer to item 2, estimated

discharge pressure and item4, EIS temp reading.

7. EEV opening should be 10% to 12%; do assist observe EIS temp reading after

gas top-up. Do carry-out short PTI from P01 to P08 test. After completion of

P08 test (Pump down test); do power OFF and re-ON.

8. As per parameter, eev opening shows 7%, suspect eev body jammed. Do assist

to Clean and service EEV body & coil orifice, follow by pushing a few times on

the spring orifice on EEV body to remove water condensation or residue at the

EEV body bellow. Refit EEV coil. Do note, water in EEV bellow will cause freeze

up during operation, causing EEV operates irregular. Aft re-installation of

EEV, kindly switch off and run SPTI till SP32 and revert us if any alarm

triggered, such as J291.

9. Do note, System gas overcharged will affect poor cooling capacity and

causing SMV to close more.

10. Do note, System gas overcharged may encounter alarm E107 or E101

intermittently after defrost termination.

11. Kindly advise, any water drains out from drain hose; after / before

completion of defrosting.

ALE101

Unit : NYKU7151501 Model : LXE10E-A26E Setpoint : -1.3c

1. Thanks for the efforts in repairing the unit.

2. Alarm E101 is HPS turns open during operation. Please assist to fix manifold gauge and check on system discharge pressure. As manifold gauge discharge pressure shows 190kpa(very low) but in display shows 1106kpa.

3. Discharge pressure may be high due to system overcharged, high ambient temperature or defective HPS. (Alarm F101 can be activated in HPS faulty).

4. Please assist to wash condenser coil for any dirt on the fins. If pressure still high or if system overcharge, please assist to purge out some gas.

5. As stated, compressor not running ? , Do assist to measure, input voltage at main circuit and output voltage at compressor contactor and Phase contactors? Is there any imbalance of 3phase voltage? If Yes, do counter check on 440v plug for any loose wire connection or burnt and any improper joint or poor contact on 440v splicing kit joint / non- standard joint on 440v power cable.

6. Do observe any voltage or amperes fluctuating or abnormal on Voltage displays, CT1 and CT displays? If yes, replace PTCT board PN: 0991050, 2139904 or 2289690.

7. Temporary isolate Compressor wires from Compressor output terminal, re-ON unit; is alarm 103 activated? If NO, please check compressor cable resistance for any open circuit or loose connection of compressor wires at compressor terminal.

Container ID: BMOU9763392, Model: LXE10E-136B2 & SP: -24C.

1. thanks for effort and feedback.

2. Alarm E101 is high discharge compressor pressure and it may due to HPT out of range, drier inlet /outlet flaring loosens or leakage, system gas low / overcharged causing mixture of air in the system or valve cores sucking of air to the system. Any repeated alarms for more than 3 times, F803 will also activated.

3. Please assist to wash condenser coil for any dirt on the fins.

4. Please fit a manifold gauge on discharge svc port and compare it with HPT display. If both pressures are difference, replace High pressure transducer.

5. Do check drier inlet/outlet flaring with oil stained / gas leakage; that causing gas leakage causing air sucking to the system causing high discharge pressure. Do purge out gas if gas level abnormal.

6. In case of box temperature continue rising after released of gas. Suspect system with a mixture of air in the system, please do replace drier, vacuum system and top up full charge of gas.

7.Do assist to provide current status of the unit with latest parameter values, -- RA, SA, DRA, DSA, HPT, LPT, CT1, CT2, AMB, DCHS, EIS, EOS, MV, SGS and EV.

Unit FSCU5724939 Model:LXE10F11B3NH with setpoint 4.4C ,

Unit NYKU7172839 Model: Daikin and with set temp of 1.0C.

1. Thanks for MAL report.

2. Alarm E101 is high discharge compressor pressure and it may due to HPT out of range, system overcharged, mixture of air in the system, valve cores sucking of air to the system, ambient temperature high or defective HPS.

3. Do assist to compare discharge manifold pressure with HPT displays. Are both discharge pressure the same? If different, possibly HPT PN: 158795J defective.

4. Please assist to fix manifold gauge and check on system discharge pressure. Based on RA 35.0C and ambient temp is 31.1C, discharge pressure on manifold gauge should be around 1800KPA TO 1850KPA. Do slowly purge some gas since discharge pressure is high.

5. Please assist to wash condenser coil for any dirt on the fins. If pressure still high or if system overcharge, please assist to purge out some gas.

6. If pressure still abnormal , do assist to deep vacuum system and to up gas as per name plate .

7. Please provide details value on display -- RA, SA, DRA, DSA, HPT, LPT, CT1, CT2, AMB, DCHS, EIS, EOS, MV, SGS and EV. Also provide suction & discharge manifold pressure.

1. thanks for the malfunction report .

2. Alarm E101 is HPS turns open during operation. Please assist to fix manifold gauge and check on system discharge pressure.

3. Discharge pressure may be high due to system overcharged, high ambient temperature or defective HPS.

4. Please assist to wash condenser coil for any dirt on the fins. If pressure still high or if system overcharge, please assist to purge out some gas.

5.If pressure still abnormal , do assist to vacuum system and to up gas as per name plate .

6. Please provide details value on display -- RA, SA, DRA, DSA, HPT, LPT, CT1, CT2, AMB, DCHS, EIS, EOS, MV, SGS and EV.

Gdays Master of “NYK Delphinus”,

Unit SZLU9600987 Model: LXE10E136A2 and with set temp of -26.1C.

1. Thanks for MAL report.

2. Do temporary isolate EIS wires and alarm E407 will be activated but just ignore alarm. do run the unit and observe if still problem exist, please follow below instruction.

3. Based on RA -21.8C and ambient temp is 30.8C, discharge pressure on manifold gauge should be around 1300KPA TO 1350KPA. Do slowly purge some gas if discharge pressure is high.

4. Do note, System gas overcharged will affect poor cooling capacity and causing SMV to close more.
5. Do assist to reset defrost timer 99hourly for any frozen setting; unit is programmed will auto defrost every 4hourly if unit is out of range.

6. In case, box temp not improving or alarms activation; do provide details value on display -- RA, SA, DRA, DSA, HPT, LPT, CT1, CT2, AMB, DCHS, EIS, EOS, MV, SGS and EV. Also provide suction discharge manifold pressure.

OUT OF RANGE/EEV COIL CAP

Unit TGHU9907263 Model LXE10E-A36A and with Set Temp of -25.0C

1. Thanks for the efforts in repairing the unit.

2. Based on RA -22.5C and if ambient temp is +28.0C; unit discharge manifold pressure or HPT display should be 1100Kpa to 1150Kpa.

3. Before gas release or top up, do assist to compare discharge manifold pressure with HPT displays. Are both discharge pressure the same? If different, possibly HPT PN: 158795J defective.

4. Do ensure EIS temp readings is cooler than EOS temp, if not compressor body will heavily frost-up and compressor body temp or DCHS display should be +70C to +98.0C. If these 2 readings are NOT match, do provide us parameter values.

5. To ensure unit in good performance; do provide details value on display -- RA, SA, DRA, DSA, HPT, LPT, CT1, CT2, AMB, DCHS, EIS, EOS, MV, SGS and EV.

6. Do assist to reset defrost timer 99hourly for any frozen setting; unit is programmed will auto defrost every 4 hourly if unit is out of range.

7.In case, prefer unit continue using EEV emergency cap; do assist to remove before unit discharge to destination port.

8.Do assist to close monitor, if any abnormalities arise

Unit CGMU4941580 Model: LXE10E-A15J and with set temp of -18.0C.

1.Thank for the feedback .

2.Base on parameters , unit EEV opening is 7% only and compressor body full of ice means EEV body jammed .

3.Do assist to use  emergency cap to open EEV body and adjust slowly .

4.Please revert outcome .

BYPASS HOSE

Container ID: HJMU6094340, Model: LXE10E-A33A & SP: 5C.

1. Thanks for the efforts in temporary repairing the unit.

2. Kindly continue used the bypass hose until POD. (Please note; Do not forget to remove the bypass hose and keep as vessel stock before the malfunction unit discharge to yard.).

3. We will arrange shore assistance at POD to properly rectify the unit.

Dear ONE.Line SINGAPORE and PSA Reefercare Team,

1. Kindly assist to plan prior to discharge the unit HJMU6094340 and arrange shore assistance to carry out necessary repair. Vessel ETA 10th July.

2. Do assist to remove the bypass hose and return to vessel personnel if found not yet being removed from the unit.

TRACE LEAKAGE

Unit KKFU6965020 Model LXE10E-A9C with set temp of -20.0C.

1. Thanks for the feedback.

2. Do assist to trace gas leakage or oil stained. Gas leakage can be from servicing core valves worn out causing leakage or drier inlet / outlet flaring worn out or loose connection or other HPT / LPT capillary tubing or joint leaking.

3. Please do check is drier outlet piping cooler than inlet piping or sweating on drier filter & piping? If yes, replace drier (suspect choked).

4. Do assist t remove EEV Emergency Cap, let the unit run without EEV coil or Emergency cap.

5. Do revert us after work done with parameter value.

1.Thank for the malfunction report .

2.Base on RA - 15.8C with ambient +43.2C, unit discharge pressure should be 1450 Kpa to 1550 Kpa . do assist to top up gas .

3. Before gas release or top up, do assist to compare discharge manifold pressure with HPT displays. Are both discharge pressure the same? If different, possibly HPT PN: 158795J defective.

4. If gas level is normal, follow up next. Do assist to observe both EIS and EOS temp readings; if EIS temp is warmer than EOS temperature and may cause compressor body heavily frost-up. Please assist to OFF the unit 3 to 5 minutes and re-ON it. Do note, compressor body heavily frost up will affect unit cooling capacity.

5. Observe again, EIS and EOS temp readings. If EIS temp still warmer than EOS, temporary isolates EIS wires, alarm 407 will activate, just ignore and not affecting unit performance.

6. Please do check is drier outlet piping cooler than inlet piping or sweating on drier filter & piping? Any oil stained or gas leakage at inlet/outlet drier flaring?  If yes, replace drier.

7. In case, box temp not improving or alarms activation; do provide details value on display -- RA, SA, DRA, DSA, HPT, LPT, CT1, CT2, AMB, DCHS, EIS, EOS, MV, SGS and EV.

8. Do assist to reset defrost timer 99hourly for any frozen setting; unit is programmed will auto defrost every 4hourly if unit is out of range.

OUT OF RANGE

Unit NYKU7173964 Model:LXE10E-A27 with setpoint -18C,

1. Thanks for the malfunction report .

2. Alarm E109 is system gas shortage due to leakage or system under vacuum. Do assist to observe is EEV assembly being covered with frost ice, EEV coil flange loosen of EEV coil shaft jammed.

3. If yes, do assist to Clean and service EEV body & coil orifice, follow by pushing a few times on the spring orifice on EEV body to remove water condensation or residue at the EEV body bellow. Refit EEV coil. Do note, water in EEV bellow will cause freeze up during operation, causing EEV operates irregular.4. Do assist to top up gas if unit gas level is too low.

5. Do assist to trace gas leakage or oil stained. Gas leakage can be from servicing core valves worn out causing leakage or drier inlet / outlet flaring worn out or loose connection or other HPT / LPT capillary tubing or joint leaking.

6. Please do check is drier outlet piping cooler than inlet piping or sweating on drier filter & piping? If yes, replace drier (suspect choked).

7. Please provide details value on display -- RA, SA, DRA, DSA, HPT, LPT, CT1, CT2, AMB, DCHS, EIS, EOS, MV, SGS and EV.

Unit NYKU7167323 Model: LXE10E-A26E and with set temp of -18.0C.

1. Thanks for MAL report.

2. Alarm E109 is system gas shortage due to leakage cause by leakage or system under vacuum or EEV assembly being covered with frost ice, EEV coil flange loosen of EEV coil shaft jammed.

3. Do assist to Clean and service EEV body & coil orifice, follow by pushing a few times on the spring orifice on EEV body to remove water condensation or residue at the EEV body bellow. Refit EEV coil by hand only. Do note, water in EEV bellow will cause freeze up during operation, causing EEV operates irregular. Please see attached for EEV servicing.

4. Do assist to top up gas if unit observe gas level is too low.

5. Do assist to trace gas leakage or oil stained. Gas leakage can be from servicing core valves worn out causing leakage or drier inlet / outlet flaring worn out or loose connection or other HPT / LPT capillary tubing or joint leaking.

6. Please do check is drier outlet piping cooler than inlet piping or sweating on drier filter & piping? If yes, replace drier (suspect choked).

7. In case same problem occurs, please provide details value on display -- RA, SA, DRA, DSA, HPT, LPT, CT1, CT2, AMB, DCHS, EIS, EOS, MV, SGS and EV. Also provide suction & discharge manifold pressure.

EMERGENCY COIL/SHORE ASSISTANCE

Good Day Master of HYUNDAI FORCE,

Unit MORU0607666 Model: LXE10E-A12C with set temp of -18.0C.

1.Thanks for the efforts for operating the unit in EV by-passed.

2. Do remove emergency coil cap before unit discharge in yard for vessel future use.

3. Do closely monitor the unit and revert if any abnormalities arise with parameter value of -- RA, SA, DRA, DSA, HPT, LPT, CT1, CT2, AMB, DCHS, EIS, EOS, MV, SGS and EV.

Good Day ONE.Line PUSAN ,KOREA Team,

Terminal: PNC (PUSAN NEWPORT COMPANY).

Kindly arrange shore assistance for unit MORU0607666 upon unit discharge in yard. Vessel ETA: 14th May.

Note: Vessel has carried out Emergency by-passed for EEV for temporary repair.

Please do remove EEV Emergency coil cap and return to vessel personnel for their future use if part not being removed by vessel technician.

Unit MORU6704642 Model: LXE10E-A15 and with Set Temp of -21.0C.

1. Thanks for the efforts in temporary repairing by using EEV emergency coil cap.

2. Do assist to remove emergency coil cap and keep as spare for future use on EEV coil breakdown onboard before unit discharge to yard.

3. We will arrange shore assistance at discharging port.

Dear ONE.Line HONG KONG Team,

Port Terminal: HIT (HONGKONG INTERNATIONAL TERMINALS )

1. Kindly assist to arrange shore assistance for unit MORU6704642, upon unit discharge to yard. Vessel ETA 14th July.

2. Do assist to remove EEV emergency coil cap and return to the vessel personnel, if found not yet been removed to the EEV body.

Unit : AMCU9264922 Model : LXE10E Setpoint : -22.0C.

1. Thanks for the feedback and confirmation.

2. Do assist to continue closely monitor and revert if any other abnormalities arise.

3. Do assist to remove emergency coil cap before unit discharge to the yard and keep as spare for future use.

4. We will arrange shore assistance at POD.

Dear CMA CGM Tanger MED Team,

1. Kindly assist to arrange shore assistance for unit AMCU9264922 upon unit discharge to the yard. Vessel ETA 14th May.

2. Do assist to remove emergency coil cap and return to vessel personnel, if found not yet been removed to the unit.

Unit FSCU5686572 model:LXE10E-A15j and with Set Temp of -22C.

1. Thanks for the efforts in repairing the unit.

2.Do assist to remove the emergency coil cap before/prior unit discharge (For ship’s spare).

3.Shore assistance will follow up repair at yard.

Gdays ONE PUSAN team,

1. Do assist to arrange shore assistance upon unit discharge to yard; Vessel ETA: 12/12/2019.

2. Do assist to remove emergency coil cap from the unit and return to Vessel Master if Master forget to remove it.

3. Please kindly find attached reefer malfunction report and take necessary action.

Note: EEV failure.

DISCHARGE HIGH

1.Thanks for the efforts and feedback.  Discharge pressure high2.Do assist to attach manifold gauge to the unit and compare the readings of HPT/LPT and manifold pressure it should be the same.3.Transfer manifold gauge high pressure hose to service port #4 (after check valve) if reading is big difference, more than 3 bars, then the check valve might chocked. Do by-pass check valve until POD, do find attached.4. If item 3 and 4 are good, the unit suspected air in the system or refrigerant with mix non condensable gas. Evacuate or vacuum all service ports and renew refrigerant. Charge based on the name plate.5.Do revert the outcome with parameter value.

EEV

Container ID: CGMU4980288, Model: LXE10E-A15J & SP: -20C.

1. thanks for effort and feedback.

2.If after defrost no water comes out at drain hose. Do assist to clean drainage hose.

3. Is compressor frosting? Check EEV body if spring orifice can be press?If not, presence of residue or icing at the EEV body bellow .Blow with air to remove residue and water at the EEV body bellow.

4.If the EEV coil orifice is too short, screw up with plier and refit EEV coil.

5. In case, if the EEV coil does not vibrate corresponding to opening change, Check EEV coil resistance ,white-red, orange -red, yellow- brown and blue-brown = 150 ohms. If one pair don't have, replace EEV coil.

6. Check evap coil by removing of 2 access panel, if evep coil some portion covered with ice. Do heating of ice with heat gun.

7. Defrost timer should be set to 99hourly.

PTI

1. Thanks for the MAL report.

2. Do assist to place back the EEV and SMV coil.

3. Do assist to initiate SPTI SP01 to SP32, do revert what is the alarm exist.

4.Please provide details values on display -- RA, SA, DRA, DSA, HPT, LPT, CT1, CT2, AMB,

DCHS, EIS, EOS, SGS, SMV and EEV.

REPLACING PT/CT BOARD

Unit CGMU4944402 Model LXE10E-A and with Set Temp of -18.0C.

1.Thanks for the MAL report.

2. Do observe display “PT” showing 380 - 440volts, “CT1” displaying total current and “CT2” displaying compressor current.  If one of the reading abnormal or fluctuating or reflecting actual current draws of the component, replace PTCT board .

3.Before replace PTCT , please cut J1 and J2 (see attach photo)

3. Replace controller with same model, if above found OK. (Suspected CPU board malfunction)

4. In case no spare on board, kindly cannibalise from any empty CMA CGM reefer (First priority). IF part been cannibalised, do provide us the container no and POD.

5. Do revert us, if any abnormalities arise with latest parameter values.

CA/02/CO2

Unit BMOU9870976  Model: LXE10E-136JC and with CA Set CO2=5% O2=5% and

ACTUAL CA level is CO2=0.1% O2=8.0%.

1.Thank for the feedback .

2.Do assist to check both evaporator panel whether leak ? if yes , temporary patching

with aluminium tape / duct tape .

3.Do assist to monitor unit and if not improve , let us know.

Unit BMOU9870976  Model: LXE10E-136JC and with CA Set CO2=5% O2=5% and ACTUAL CA level is CO2=0.2% O2=7.7%.

1.thank for the feedback and do assist to monitor unit .

2.Currently both CO2 and O2 are slowly improved .

Unit BMOU9870976  Model: LXE10E-136JC and with CA Set CO2=5% O2=5% and ACTUAL CA level is CO2=0.1% O2=9.0%.

1. Thanks for the efforts and feedback.

2. Please do observe again under structure of T-floor drain plug / Kazoo, 2each at rear and front bottom side; are there any cool air leakage or water dripping? If yes, do assist to re-seal it.

3. Please do check vacuum pump operation. Do revert after work perform.

Unit BMOU9870976  Model: LXE10E-136JC and with CA Set CO2=5% O2=5% and

ACTUAL

CA level is CO2=0.1% O2=7.4%.

1. Thanks for the fforts and feedback. Please cycle unit power off and on.

2. Please hold “Select Key” once after power cycle to navigate to PTI menu.

3. Scroll through PTI menu until reach “CA TEST” option. Activate CA Test, the test

will take approximately 20-30 minutes.

4. If Test is successful and shows “GOOD” please cycle power off and on and allow

unit to run in CA mode. Please revert with O2 and CO2 readings.

5. If  CO2 reading is above 0.1%, continue monitoring of container and no further

action to be taken.

6. In the event CA Test fails and alarm will be shown. Please revert with alarm

number.

Unit SCEU9000720  Model: LXE10E-136HK and with CA Set CO2=5% O2=5% and

ACTUAL

CA level is CO2=1.6% O2=10%.

1. Thanks for the efforts and feedback.

2. Base on the report, CO2 is gradually increase and O2 is gradually decrease its a

slowly progress it will take more days to reach the setting.

3. Do assist to closely monitor, do observe within 24hours then revert us with CO2 and

O2 Actual level.

CA PTI TEST

Unit BMOU9872177 Model: LXE10E- 136JC and with CA setting CO2 5% O2 5% and ACTUAL CA level CO2 0.0% O2 5.8%.

1. Thanks for the feedback. Please cycle unit power off and on.

2. Please hold “Select Key” once after power cycle to navigate to PTI menu.

3. Scroll through PTI menu until reach “CA TEST” option. Activate CA Test, the test will take approximately 20-30 minutes.

4. If Test is successful and shows “GOOD” please cycle power off and on and allow unit to run in CA mode. Please revert with O2 and CO2 readings.

5. If CO2 reading is above 0.1%, continuous monitoring of container and no further action to be taken.

6. In the event CA Test fails and the alarm will be shown. Please revert with alarm number.

H006/PTI

Unit BMOU8703526 Model: LXE10E-A32B9 and with Set Temp of +20.0C.

1. Thanks for the MAL report.

2. Do assist to Turn OFF/ON Unit" counter check "H006" Setting  should be (H006 - 3) on display; Press and Hold "SELECT Key" shown (FPTI),  Press and Hold "SELECT Key" shown (USDA), Press and Hold "SELECT Key" shown (DECOS d),  Press and Hold "SELECT Key" shown (Degree C/F), Press "Set Key" to find (H001 - 3) (H002 - 2) (H003 - 2) (H004 - 2) (H005 - 3) (H006 - 3).

3. If H006 show value different from the item #2, Do assist to Reset; Press "Arrow up / Arrow Down Key" then press "Enter Key".

4. After re-setting "Turn OFF C.B" then "Turn On C.B" after 5 seconds to record controller changes.

5. Sometimes RA,SA/DRA,DSA have a problem or change values can trigger also the Alarm H006.

6. Please provide details value on display -- RA, SA, DRA, DSA, HPT, LPT, PT, CT1, CT2, AMB, DCHS, EIS, EOS, SMV, SGS and EV.

F603

Container ID: FSCU5683146, Model: LXE10E A15J9 & SP: -21C.

1. Thanks for effort and feedback.

2. For Alarm E707, do assist to measure voltage at input / output of main breaker. Possible voltage abnormal. Do assist to counter check, any loose wire connection or 440v plug partial burnt? Do counter on splicing kit joint on 440v power cable, possibly poor contact or open circuit.

3. Do switch to another good power point, possibly internal receptacle wire burnt.

4. Do assist to advice, is anyone of phase contactor energies? If No, do assist to measure the resistance of both phase contactors and compressor contactor; is there any low resistance? If Yes, replace Contactor PN: 1198917 or 119891J. Compressor / phase contactor --- 3.7Ω    124149J or 1241493 phase contactor. PN: 119891J or 1198917 compressor contactor.

5. Please observe display “PT” showing 380 - 440volts, “CT1” displaying total current and “CT2” displaying compressor current.  If one of the reading abnormal or fluctuating or reflecting actual current draws of the component, replace PTCT board PN: 0991050 or PN: 2139904 or PN: 2289690..

6. If neither one of phase contactors energies. Do measure any 24Vac at wire 143. If  have 24Vac, measure wire 153 and 104. If have 24Vac, then PCC1 have to energies.

7. If no 24Vac at wire 143 and 104; do measure Voltage at Wire 145 and 104. If have 24Vac, measure wire 154 and 104. If have 24vac, then PCC2 have to energies.

8. If one of Phase Contactor is energies and compressor contactor NOT energies. Do assist to measure any 24Vac at wire 133 of compressor contactor (CC) and wire 104? Wire 155 (CC) and wire 104 (negative wire). May temporary bypass HPS wires, if No voltage input. Release gas if discharge pressure higher than 2200Kpa, because no protection on compressor as HPS wires being bypassed.

9. For F603, do assist to check controller DECOS setting which is DECOS 3d.

10. Do assist to Turn OFF Circuit breaker Unit and ON" check decos Setting  should be "d"  on display; after unit starting, immediately Press and Hold 3 seconds "SELECT Key" shown (FPTI),  Press and Hold "SELECT Key" shown (USDA), Press and Hold "SELECT Key"  and press ARROW keys to select (DECOS d),  and turn OFF unit after selecting 3D.

11. Measure SMV resistance 113 ohms each pair  white-black and blue-yellow, if resistance out range replace SMV coil. Do check SMV board fuse if blown. Do measure power voltage supply from transformer to terminal board it should be 24v, 13v, and 24v.

12. Before 10 seconds after unit staring, measure smv voltage 24v of each pair white-black and blue-yellow.

13. If controller confirmed out of order, do follow attached instruction for controller emergency by-Pass.

14. By performing emergency by-pass, be careful temperature cannot be controlled by unit itself unit may drop below set temp, do read actual temperature by thermometer and OFF the unit if required temperature has reached and ON the unit if temp become higher.

15. If DECOS 3d controller not available on board, do replace controller kit 2388799 with ACC KIT FOR CONTROLLER 2179351.

Unit MORU0602319 Model: LXE10E-A12C and with Set Temp of -0.5C.

1. Thank for MAL report.

2. Do assist to counter check again, any loose wire connection or 440v plug partial burnt? Do counter on splicing kit joint on 440v power cable, possibly poor contact or open circuit. Do switch to another good power point, possibly internal receptacle wire burnt.

3. Do assist to advice, is anyone of phase contactor energies? If No, do assist to measure the resistance of both phase contactors and compressor contactor; is there any low resistance? If Yes, replace Contactor PN: 1198917 or 119891J. Compressor / phase contactor --- 3.7Ω    124149J or 1241493 phase contactor. PN: 119891J or 1198917 compressor contactor.

4. Please observe display “PT” showing 380 - 440volts, “CT1” displaying total current and “CT2” displaying compressor current.  If one of the reading abnormal or fluctuating or reflecting actual current draws of the component, replace PTCT board PN: 0991050 or PN: 2139904 or PN: 2289690..

5. Do remove/clean all socket of controller harness within 20 mins and refit back and cycle the unit.

6. kindly check any moisture present at CPU board socket.

7. after performing item #2 to #6 and unit still ALE707 activated, do replace controller. (Suspected CPU board malfunction)

8. In case no spare on board, kindly cannibalize from any dry/empty ONE line reefer (First priority). If part been cannibalized, do provide us the container no and POD with ETA.

9. If still no empty / dry to cannibalize, do switch OFF unit & temporary set SMV under emergency mode to open SMV valve (Refer to image). try to run the unit.

10. Do revert after work perform.

AL707

Unit CGMU5296069 Model LXE10E- 136J2 with set temp of +20.0C.

1. Thanks for the MAL report. Do troubleshoot and carry out necessary repair when the weather permit.

2. Kindly Turn off unit CB and check any loose or poor contacts of controller sockets.

3. Do remove and refit at PTCT connection from controller and do observe any loose contacts of PTCT board.

4. Do remove and refit controller harness connected to I/O board.

5. Do measure Circuit breaker output voltage 3 phase 440 v. If voltage imbalance or single phase, kindly check partial burn't of male plug, cable loose or poor contacts of splicing joints. Or may transfer to other power source outlet. If alarm still active do replace PTCT board 2289690

6. Observed SA and RA temp dropped, kindly ambient temp or kindly check any moisture present at CPU board sensor socket.

7. Do revert soonest if repair carried out, if alarm still appear, shore assistance shall be arrange

Unit CGMU4952537 Model: LXE10E-A15J and with Set Temp of +2.0C.

1. Thanks for the MAL report.

2. Since unit block out, do remove all socket of controller harness within 20 mins and refit back and cycle the unit.

3. kindly check any moisture present at CPU board socket.

4. after performing item 2 and unit still block out, do replace controller. (Suspected CPU board malfunction)

5. In case no spare on board, kindly cannibalize from any empty CMA CGM reefer (First priority). IF part been cannibalized, do provide us the container no and POD/ETA.

6. Do revert after work perform.

Unit NYKU7167200 Model: LXE10E-A26 and with Set Temp of -18C,

1. Thank you for the malfunction report.

2. This unit NYKU7167200 ( below 100series) uses display with PN : 2275095 and BMOU9715429(above 100series) uses display PN : 1869769.

3. We suggest to swap the display back to its original cannibalised unit and fix the defective part back to BMOU9715429 before unit to be discharged.

4. Since discharging port Kobe, ETA 16/1 is only one day after Tokyo, we suggest repairs to be carried out at discharging port.

ALE403/RETURN SENSOR FAILURE

Unit BMOU8717520 Lxe10E-136B2 with setpoint -20C.
1.Thank for the malfunction report .
2.For alarm E407 , just ignore alarm and repair will follow up next PTI.
3.For alarm E403 , There are 2 return sensors, one is Control [RA] and another is Datacorder RA [DRA]. Isolate the wires of Control or Datacorder RA one by one, it is linked shorted and not both RA failure.
4. Please try to remove on one of RA or DRA sensor and observe on screen is sensor reading OK. If no, refit the remove sensor and isolate the other sensor. Is sensor reading OK?
5. Before carry out isolation of sensor wires; do assist to check sensors mounting terminal for any loose contact and re- secure it.
6. Return sensor will be replace after box return empty for next PTI.

Unit BMOU8708997 Model: LXE10E136A1 and with set temp 15.0C,

1. Thanks efforts & feedback. do switch OFF unit & follow below instruction.

2. Do try to counter check if unit have spare sensor same as attached photo. If have, do assist to cut the wires of return sensor at CN43 7 & 8 then try to connect the spare return sensor with blue/blue colour marking then observe unit.

3. If don't have spare, do temporary isolate wires of return sensor since out of range & monitor the return reading at DR or (datarecorder return).

4. After re-setting "Turn OFF C.B" then "Turn On C.B" after 5 seconds to record controller changes.

5. Do revert outcome with details value on display -- RA, SA, DRA, DSA, HPT, LPT, PT, CT1, CT2, AMB, DCHS, EIS, EOS, SMV, SGS and EV.

E401/E402

Unit SZLU9607370 Model: LXE10E-136A2 and with Set Temp 20.0C,

1. Thanks for effort and feedback.

2. If alarm E401/E402 activated again, There are 2 supply sensors, one is Control [SA] and another is Datacorder SA [DSA]. Isolate the wires of Control or Datacorder SA one by one, it is linked shorted and not both SA failure.

3. Please try to remove on one of SA or DSA sensor and observe on screen is sensor reading OK. If no, refit the remove sensor and isolate the other sensor. Is sensor reading OK?

4. Before carry out isolation of sensor wires; do assist to check sensors mounting terminal for any loose contact and re- secure it.

5. Supply sensor will be replace after box return empty for next PTI.

1. Thank you for the mal report.

2. Please note; NOT both SS / DSS sensors are defective; it is link together some due to one of Supply sensor shorted.

3. Do assist to check the wire connection for both sensors, it could be loose connection / wire shorted.

4. Do assist to remove and refit the wire and restart the unit. If confirmed one of the supply sensor defective, it can be ignored since one of the sensor is still working.

5. If both sensor reading is faulty, SA sensor can be replaced at sampling port.

Unit : NYKU7172551 Model : LXE10E- A27 Setpoint : 6.0C 1. Thank you for the mal report.
2. There are 2supply sensors, one is Control [SS] and another is Datacorder SA [DSA]. Isolate the wires of Control or Datacorder SA one by one, it is linked shorted and not both SA failure.
3. Please try to remove on one of SA or DSS sensor and observe on screen is sensor reading OK. If no, refit the remove sensor and isolate the other sensor. Is sensor reading OK? In case, both Supply failure, may use ambient sensor.
4. Or, please remove the ambient sensor sensing probe and move it sample port. And ambient sensor wires remove and connected to terminal which sensor terminal is defective or on Datacorder Supply terminal (DSS). Just ignore ambient sensor alarm will not affect unit performance. Carry out this repair when BOTH SA SENSOR are out or defective.
5. Before carry out isolation of sensor wires; do assist to check sensors mounting terminal for any loose contact and re-secure it.
6. There is not necessary to arrange shore assistance and not direct access to replace Supply sensor. Unit controller will auto switch to RA sensor for Control. 
7. Supply sensor will be replaced after box return empty for next PTI.

Container ID: NYKU7159272, Model: LXE10E-A26E &  SP: 0C.

1. Thanks for the efforts in temporary repairing the unit.

2. Since SA wires is already disconnected, just ignore the Alarm E401. And repair will follow-up during next PTI.

3. Do get the supply reading value into DSA. (Note; SA and DSA are mounted in same location and same position.)

4. Do continue to closely monitor and revert if any other abnormalities arise.

Unit TCLU1902815 Model: LXE10E136B4R and with set temp 15.0C,

1. Thanks for efforts in temporary repairing the unit.

2. Do continue monitor supply temperature via DSA reading since found SA sensor was defective.

3. Do continue monitor till POD & revert if any other abnormalities arise with latest parameter values;

4. For faulty supply sensor (SA), repair will follow-up during next PTI.

Unit BMOU9740967 Model LXE10E136A2 with set temp of +5.0C.
1.thanks for feddback.
2.Do assist to check the SS is +10.4C remove and refit the socket of sensor on base diagram.
3.do advice if any spare controller board see the sensor spare color green, kindly use spare supply sensor with marked 2 color green can you replace the old sensor to new sensor spare.
4.Please do revert outcome.

Unit BMOU9740967 Model LXE10E136A2 with set temp of +5.0C.
1.Thanks for the Mal report. Alarm E401 and E402, one of the supply sensor got failure.
2. Kindly OFF the unit then ON, after 18 sec. of controller preparation, kindly check current Supply Temp and DS temp, if one sensor reading much too far from return temp or kindly measure resistance of each sensor and if found one sensor out of range, kindly use spare supply sensor with marked 2 color green, do find attached image for you reference.
3. Do revert outcome with parameter value.

AL E401

Unit MORU0711032 Model: LXE10E-A37 and with Set Temp of -25.0C

1. Thanks for MAL report.

2. For alarm E407 , do assist to isolate EIS wires alarm 407 will activate just ignore and not affecting unit performance. repair will follow up during requested next PTI.

3. Before carry out isolation of sensor wires; do assist to check sensors mounting terminal for any loose contact and re- secure it.
4. Do assist to reset defrost timer 99hourly for any frozen setting; unit is programmed will auto defrost every 4hourly if unit is out of range.

5. In case, box temp not improving or any another alarms exist; do provide details value on display -- RA, SA, DRA, DSA, HPT, LPT, CT1, CT2, AMB, DCHS, EIS, EOS, MV, SGS and EV.

Container ID: BMOU8708955 Model: LXE10E-136A1 & SP: 4.0C.

1. Thanks for the efforts in repairing the unit.

2. Please note; NOT both SS / DSS sensors are defective; it is link together some due to one of Supply sensor shorted.

3. Just isolate CN43-9 and observe; is RA sensor temp appears on screen, if NO, refit CN43-9 and isolate CN43-11. Is Datacorder Supply Sensor temp appears on screen? Once, there is supply sensor temp appear on screen, temporary isolate another wire. If both have reading after isolated. Do compare with normal temp readings.

4. Do assist to revert the outcomes of repair.

5. Do note, new Daikin Software program will auto switch to another Sensor control. If box temp under Control and in-range, no further servicing require till discharging Port.

MOISTURIZE

Unit : SZLU3637119  Model : LXE10E136GC Setpoint : -18.0C

1. Thank you for the parameters value. As per readings, unit seems to be normal expect compressor temp. high and discharge press. high due to high ambient temp.

2. Do check whether is there any air leak from air-vent panel. If yes, kindly open ef panel and check whether any ice formation. Do tighten the wing bolt and use aluminium tape to arrest the leak if any.

3. If box temp not improving, suggest to vacuum system and top up r134a accordingly to required kgs, as stated in container's nameplate. Do check whether valve core leaking during this process.

4. Do monitor the unit closely and revert us with parameters.

HIGH TEMP

Unit : SZLU3637119  Model : LXE10E136GC Setpoint : -18.0C 1. Thank you for the malfunction report.
2. Alarm E109 is caused by deep low suction pressure and may due to system gas low caused by gas leakage, liquid line solenoid valve restriction or choked or no power to liquid solenoid coil or liquid receiver’s strainer may choked or system choked or drier choked, EEV’s orifice freeze with ice or defective of EEV coil.
3. Please check and measure EEV coil for any loose connection or low coil resistance & EEV body’s orifice for any ice freeze up? Resecure EEV coil and if suction pressure failed to rise after securing or low resistance, replace EEV Coil PN: 1890819.
4. Digital EEV coil 46 +/-3 OHM [White - Red: 46Ω, Orange - Red: 46Ω, Yellow - Brown: 46Ω and Blue - Brown: 46Ω]. Do note, EEV coil may have 5 or 6wires.
5. Do assist to trace gas leakage or oil stained. Gas leakage can be from servicing core valves worn out causing leakage or drier inlet / outlet flaring worn out or loose connection or other HPT / LPT capillary tubing or joint leaking.
6. Please do check is drier outlet piping cooler than inlet piping or sweating on drier filter & piping? If yes, replace drier.
7. Please check is receiver strainer is choked? Replace strainer.
8. Alarm E101 is high discharge compressor pressure and it may due to HPT out of range, drier inlet /outlet flaring loosens or leakage, system gas low / overcharged causing mixture of air in the system or valve cores sucking of air to the system. Any repeated alarms for more than 3 times, F803 will also activated.
9. Since ambient is high causing discharge pressure to increase, alarm e101 triggered.
10. Temporary action for high ambient temperature, wash condenser coil with fresh water, place extra blower/ventilation and place running water over compressor for a while, it may help for the time being.
11. Do assist to purge out gas if discharge pressure readings too high. Current pressure is too high. Do purge out some gas.
12. To ensure unit operates in good condition, please provide details value on display -- RA, SA, DRA, DSA, HPT, LPT, PT, CT1, CT2, AMB, DCHS, EIS, EOS, MV, SGS and EV.

EEV

Unit NYKU7111617 Model: LXE10E-A17 and with Set Temp of +23.9C.

1. Thanks for the efforts and feedback.

2. Do assist to Clean and service EEV body & coil orifice, follow by pushing a few times on the spring orifice on EEV body to remove water condensation or residue at the EEV body bellow. Refit EEV coil hand tight only. Do note, water in EEV bellow will cause freeze up during operation, causing EEV operates irregular.

3. After cleaning of EEV coil, does EEV body orifice can be push freely? If not, EEV orifice stuck or blockage.

4. Do assist to counter check EEV Coil resistance it should be 150 ohms (White-Red) (Orange-Red) (Yellow-Brown) (Blue-Brown), if Below 150 ohms do replace EEV coil PN: 2238128.

5. Turn "ON C.B" observe EEV coil orifice needle by holding your hands before compressor running, it will rotate (clockwise and counter clockwise) when EEV coil energize.

6. If  EEV coil energize it feel vibrating or movement of motorize EEV coil orifice needle. If vibration is present and orifice needle not rotating, possibly EEV coil stuck up or orifice needle choked. Do assist to replace EEV coil if found faulty.

7. do provide details value on display after unit running in 30 minutes -- RA, SA, DRA, DSA, HPT, LPT, CT1, CT2, AMB, DCHS, EIS, EOS, SMV, SGS and EEV.

Unit TGHU9942449 Model: LXE10E-136A with setpoint -18c ,

1. Thanks for feedback.

2. Do assist to Clean and service EEV body & coil orifice again, follow by pushing a few times on the spring orifice on EEV body to remove water condensation or residue at the EEV body bellow. Refit EEV coil by hand tight only. Do note, water in EEV bellow will cause freeze up during operation, causing EEV operates irregular. Do find attached EEV servicing.

3. Kindly advise, any water drains out from drain hose; after / before completion of defrosting.

4. If no water coming out from drain pipe, suspect drain pan iced up , causing blockage, thus temp unable to pull down.

5. Shore assistance already arrange at discharging port.

EEV/PTI

Unit NYKU7147923 Model: LXE10E-A26E and with Set Temp of -18.0C.

1. Thanks for the MAL report.

2. Base on the parameter given DC 117.3C and EIS -22.3C possibly EEV body partially chocked or system gas leakage.

3.  Do carry-out Short PTI from P01 to P08 test. After completion P08 Pump down test, do power OFF and ON. These is to initial EEV opening.

4. In case, Compressor body temp still remain more than 100C. Do assist to Clean and service EEV body & coil orifice, follow by pushing a few times on the spring orifice on EEV body to remove water condensation or residue at the EEV body bellow. Refit EEV coil. Do note, water in EEV bellow will cause freeze up during operation, causing EEV operates irregular.

5. Do assist to reset defrost timer 99hourly for any frozen setting; unit is programmed will auto defrost every 4hourly if unit is out of range.

5. In case, box temp not improving or alarms activation; do provide details value on display -- RA, SA, DRA, DSA, HPT, LPT, CT1, CT2, AMB, DCHS, EIS, EOS, MV, SGS and EEV.

SMV/DROP BELOW SETPOINT

Unit NYKU7111617 Model: LXE10E-A17 and with Set Temp of +23.9C.

1. Thanks for the MAL report.

2. Do assist to measure SMV coil Voltage approx; 24 volts, ( Blue-Yellow / Black-White). If no output voltage out from I/O board, possibly I/O board faulty. Do replace Controller.

3. In case no spare on board and no available empty reefer to cannibalize. Do assist to use Emergency Magnet, remove SMV coil and place emergency magnet to coil mount section, turn emergency magnet in the close direction (clockwise). do ensure that the valve is fully close. (There is a small clicking sound.)

4. After closing the SMV do assist to rotate two times the emergency magnet to open direction (counter clockwise) the re start the unit.

5. If the operation is in vacuum state do continue rotate two times the emergency magnet to open direction (counter clockwise) do continue the process if the operation is in vacuum state.

6. If the SA Temp drop again, do assist to rotate one time the emergency magnet to close direction (clockwise) do continue the process if the operation is SA Temp drop to Set Temp.

7. Do revert outcome.

HIGH AMBIENT

Unit BMOU9872201 Model: LXE10E-136JC with Set Temp of +12.0C.

1. Thanks for MAL report.

2. Temperature slowly pull down due to high ambient.(55.8C)

3. Do assist to continue provide portable fans to add more cooling in condenser coil.

4. Do continue Wash condenser to remove blockage or any obstruction of condenser coil fins.

5. Do closely monitor and revert if any abnormalities arise with latest parameter values.

1. Thanks for the effort repairing the unit.

2. Since high ambient temp, unit will take longer time for temp to pull down gradually.

3. Below troubleshoot to be maintained for proper ventilation.

4. Do provide us the latest parameter of the unit so we can guide further.

Unit CAIU5401841 Model: LXE10E-A36A and with set temp of -24.0C.

1. Thanks for efforts and MAL report.

2. When ambient temperature becomes more hot, do assist to continue add more portable fan 1 for compressor motor and 1 for condenser coil

to add more cooling.

3. Wash condenser to remove blockage or any obstruction of condenser coil fins.

Unit CGMU5189121 Model: LXE10E-136J2 and with set temp of -22.0

1. Thanks for MAL report.
2. Suspect high ambient affect the unit process. As temporary action for high ambient temperature, do assist to put two portable fan still need to use one for compressor and one for condenser coil, it may help for the time being.

3. Do assist to Wash condenser to remove blockage or any obstruction of condenser coil fins.
4. If temperature still no improvement during next monitoring day; do provide details value on display -- RA, SA, DRA, DSA, HPT, LPT, CT1, CT2, AMB, DCHS, EIS, EOS, MV, SGS and EV.
5. Do assist to reset defrost timer 99hourly for any frozen setting; unit is programmed will auto defrost every 4hourly if unit is out of range.

USDA APPEARED/REPLACE CONTROLLER

Unit CGMU5345076 Model: LXE10E-136J2 and with Set Temp of -18.0C.

1. Thanks for the efforts and feedback.

2. When replacing controller with new one, controller configuration setting need to reset follow below instruction;

- Since "USDA" appear press "Arrow Up or Arrow Down Key" to set into "OFF" position press "ENTER Key" to confirm setting.

- Press and Hold "CHART Key" until "DTMS" appear on display then press "Arrow Up or Arrow Down Key" to set into "OFF" position press "ENTER Key" to confirm setting.

- Press "SET Key", "DHU" will appear on display then press "Arrow Up or Arrow Down Key" to set either "OFF" or "ON" position press "ENTER Key" to confirm setting. If Humidity required to the unit "DHU" set to "ON", if Humidity not required "DHU" set to "OFF" then press "ENTER Key" to confirm setting.

- Press and Hold "SELECT Key" until "DECOS" appear on display, do revert what is "Decos" setting on display either E, F, G, H, J.

- Press "SET Key", "REC SEN" will appear on display then press "Arrow Up or Arrow Down Key" to set into "ON" position press "ENTER Key" to confirm setting.

- Press and Hold "SELECT Key" until "CHARTLST" appear on display then press "Arrow Up or Arrow Down Key" to set into "OFF" position press "ENTER Key" to confirm setting.

- Press "SET Key", it will appear on display either "C or F", if display show "F" press "Arrow Up or Arrow Down Key" to set into "C" position press "ENTER Key" to confirm setting.

- Press and Hold "SELECT Key" until "ID" appear on display then press "ENTER Key", input the CGMU5345076 then press "ENTER Key" to confirm setting.

- Power OFF Circuit Breaker then "ON" after 5 seconds to save controller changes.

3. Do revert outcome.

REHEAT

Unit CGMU5451397 Model: LXE10E-136JA and with Set Temp of +17.0C and Humidity setpoint of 65%.

1. Thanks for feedback.

2. Do assist to confirm if REHEAT setting to "ON". kindly follow below instruction to set reheat setting to ON.

a) “Off” and then “ON”, during unit on initial operation; press & hold “select” for 3sec, LCD with display FULL PTI, short PTI and Manual check.

b) Press & hold “select” for 3sec, display will show “USDA” and “DHU”.

c) Press & hold “select” for 3 sec. Display will show “A or B or C or D or E or F or G or H or J” of Decos III configuration prefix.

d) Press “up & down” key till display show “J” (Note: This unit model controller is DECOS III J). Press “enter” to accept.
e) Press "SET Key", until "REHEAT" will appear on display then press "Arrow Up or Arrow Down Key" to set into "ON" position press "ENTER Key" to confirm setting.

f) “OFF” and follow by “ON” the unit main circuit breaker (Note: Must OFF/ON unit’s breaker”). The controller will then be accepting configuration changed.

PLATFORM

Unit AMCU9266000 Model: LXE10E-A15J and with Set Temp of +10.0C.

1. Thanks for the MAL report.

2. Kindly confirm, vessel crew are not possible to repair due to stowage position is third tier?

3. It will take 2 to 3 days to reach the next port, if have some idea can be used to reach and rectify the problem or use any platform so that unit cargo will save and inside box temp not rise up.

4. If Vessel crew cannot possible to reach and rectify the problem, please revert so that we can ask and arrange shore assistance at next port of call.

COMPRESSOR MOTOR ICE UP

Unit : TGHU9907495 Model : LXE10E-A36A Setpoint : -20.0C.
1. Thanks for the efforts and feedback.
2. Since compressor still ice up . do assist to perform SPTI 01 to 10 .
3.If still ice up , perform SPTI again .
4.To ensure unit operates in good condition, please provide details value on display -- RA, SA, DRA, DSA, HPT, LPT, CT1, CT2, AMB, DCHS, EIS, EOS, MV, SGS and EV.
5. If comp still ice up, do replace EEV coil and Vacuum the system top up gas as per nameplate.

NO POWER

1. Thanks for the valuable feedback and efforts in trouble shooting the unit.

2. Do assist to measure voltage at input / output of main breaker. Possible voltage abnormal. Do assist to counter check, any loose wire connection or 440v plug partial burnt? Do counter on splicing kit joint on 440v power cable, possibly poor contact or open circuit.

3. Do switch to another good power point, possibly internal receptacle wire burnt.

4. Do assist to advise, is anyone of phase contactor energies? If No, do assist to measure the resistance of both phase contactors and compressor contactor; is there any low resistance?.

5. Please observe display “PT” showing 380 - 440volts, “CT1” displaying total current and “CT2” displaying compressor current.  If one of the reading abnormal or fluctuating or reflecting actual current draws of the component, suspect PTCT board faulty.

6. If neither one of phase contactors energies. Do measure any 24Vac at wire 143. If  have 24Vac, measure wire 153 and 104. If have 24Vac, then PCC1 have to energies.

7. If no 24Vac at wire 143 and 104; do measure Voltage at Wire 145 and 104. If have 24Vac, measure wire 154 and 104. If have 24vac, then PCC2 have to energies.

8. If one of Phase Contactor is energies and compressor contactor NOT energies. Do assist to measure any 24Vac at wire 133 of compressor contactor (CC) and wire 104? Wire 155 (CC) and wire 104 (negative wire). May temporary bypass HPS wires, if No voltage input.

10. Do counter check on compressor junction box, is there any terminal wires burnt or loose connection?

11. Once, unit in operation; do provide details value on display -- RA, SA, DRA, DSA, HPT, LPT, AMB, DCHS, CT1, CT2, EIS, EOS, MV, SGS and EV for our reference.

12. E707 - Instantaneous voltage failure shutdown . Power supply voltage failure shutdown during operation (greater than 535V, less than 334V).

CANNIBALIZED EVAP MOTOR

Unit CGMU6503197 Model: LXE10E-149A2 and with Set Temp -25.0C.

1. Thanks for the MAL report.

2. Since no spare EF motor on board, kindly proceed to cannibalize EF motor from unit CGMU3000028 empty reefer.

3. If part has been cannibalized, kindly provide also ETA of POD for unit CGMU3000028 empty reefer. So that, we will arrange shore assistance upon unit discharge to yard.

4. Do assist to place the defective EF motor to CGMU3000028 empty reefer so, they will discharge together and isolate the wires to avoid wires touching the metal/body of unit.

5. Please revert outcome.

EVAP MOTOR

Unit AMCU9267541 Model: LXE10E-A15J and with Set Temp of -22.0C.
1.Thanks for the efforts and feedback.
2. Do measure resistance of each evap motor, isolate and separate evap motor wires at EF low (EFL) and high speed (EFH) contactor, measure resistance of each phase of EF motor (brown to yellow=?, yellow to blue=? and brown to blue=? for high speed approx.  17 to 18 ohms each phase), (red to white=?, white to black=? and red to black=? for low speed, 113 to 114 ohms) refit wiring connection at EF contactor if no abnormalities.
3.EVAP manual checking. Turn On the unit, press immediately SELECT key for 3 seconds until FPTI comes out, press ARROW keys to select MCHECK and press enter and select EFL/EFH AMP by selecting, will run EF motor (Do find attached for operation procedure.) Check EF motor for correct rotation, air blow must be in downward direction, measured current low speed ef motor should be  1.0 to 1.2 amperes and high speed 3.0 to 3.2 amperes should be the same amp reading in controller.
4. If one EF motor with infinite resistance or out of range resistance or high ampere, do replace EF motor.
5. Do revert outcome with value details.

ALF809(III G)

Unit CGMU5346684 Model: LXE10E-136G (DECOS G) and with Set Temp 3.0C.
1. Thanks for the efforts and feedback.
2. Alarm F809 :- EEV opening error or refrigerant shortage. Alarm F809 active for more than 3 times, E809 will also activated.
3. Do assist to check EEV coil is secure properly and resistance of EEV Coil. If resistance low, replace it. PN: 1890819 or 2266851.
4. Do clean & Clear EEV assembly frost if being covered with frost or ice. Ensure EEV coil fully secure where coil dip pin click on EEV body dip hole.
5. Digital EEV coil 46 +/-3 OHM [White - Red: 46Ω, Orange - Red: 46Ω, Yellow - Brown: 46Ω and Blue - Brown: 46Ω].
6. Please do check is drier outlet piping cooler than inlet piping or sweating on drier filter & piping? Any oil stained or gas leakage at inlet/outlet drier flaring?  If yes, replace drier.
7. If gas level is normal, follow up next. Do assist to observe both EIS and EOS temp readings; if EIS temp is warmer than EOS temp. Please assist to OFF the unit 3 to 5 minutes and re-ON it.
8. Observe again, EIS and EOS temp readings. If EIS temp still warmer than EOS, temporary isolates EIS wires, alarm 407 will activate, just ignore and not affecting unit performance.
9. To ensure unit operates in good condition, please provide details value on display -- RA, SA, DRA, DSA, HPT, LPT, CT1, CT2, AMB, DCHS, EIS, EOS, MV, SGS and EV.

Unit : CGMU5302599 Model : LXE10E-136J2 Setpoint : -20.0C
1. Thanks for the efforts in repairing the unit.
2. Before gas release or top up, do assist to compare discharge manifold pressure with HPT displays. Are both discharge pressure the same? If different, possibly HPT PN: 158795J defective.
3. Do ensure EIS temp readings is cooler than EOS temp, if not compressor body will heavily frost-up and compressor body temp or DCHS display should be +70C to +98.0C. If these 2readings are NOT match, do provide us parameter values.
4. Do assist to reset defrost timer 99hourly for any frozen setting; unit is programmed will auto defrost every 4hourly if unit is out of range.
5. EEV opening should be 10% to 12%; do assist observe EIS temp reading after gas top-up. Do carry-out short PTI from P01 to P08 test. After completion of P08 test (Pump down test); do power OFF and re-ON.
6. Kindly advise, any water drains out from drain hose; after / before completion of defrosting.
7.  In case, box temp not improving or alarms activation; do provide details value on display -- RA, SA, DRA, DSA, HPT, LPT, CT1, CT2, AMB, DCHS, EIS, EOS, MV, SGS and EV.

LEAK SEVERE

Unit KKFU6580373 Model: LXE10E-A9 and with Set Temp 0.0C,

1.Thanks for MAL report.

2.Since unit been top up refrigerant, do assist to trace gas leakage or oil stained. Gas leakage can be from servicing core valves worn out causing leakage or drier inlet / outlet flaring worn out or loose connection or other HPT / LPT capillary tubing or joint leaking.

3.Do continue monitor gas level and top up gas as when necessary until discharging port 21th April?

4.Do assist to how severe is the leakage or how much was top up and lasted on how long for 2nd top up? Do check available gas on onboard?
5. Do assist to advice, how many containers on top of this faulty reefer?

Unit CGMU4943319 Model: LXE10E-A15J and set temp: -20.0C.
1. Thanks for the feedback.
2.If no spare coupler , Do assist to find CMA /APL frozen mode reefer unit and please assist to perform air ducting on unit. For temporary measure, do assist to use air vent ducting from EF panel and to another good Frozen unit (Same Operator).  Faulty unit just operates EF Fan only.
3. Enclosed sample of temporary air cooling ducting. Do assist to isolate compressor wires from output compressor contactor and CF fan wires from output CF contactor.
4. On temporary measure on the faulty reefer, remove EF panel (Place safe in cable compartment) and assist to fit an exhaust fan ducting cooling air hose between this faulty reefer to another good frozen unit nearby (Same Operator). Ensure to duct tape to avoid cool air leaking from side.
5, The faulty reefer will temporary operates with EF high Fan getting cool air from good frozen unit.
6. Choose a good Frozen mode unit nearby (Same Operator), temporary remove EF panel (Keep safe in cable compartment) and duct tapes to avoid cool air leakage.
7. Do assist to provide daily both units box temp for our reference. Please revert.

RECOVER REFRIGERANT

Container ID: BMOU9719846 Model: LXE10E-A15 & SP: +15.6C.
1. Thanks for the efforts and feedback.
2. Do assist to recover/release the gas on the system then perform within 1hour deep vacuum all service port but do ensure while vacuum running the other service port should close with port cap to avoid system moisturize.
3. After evacuating, do observe the manifold gauge within 20 minutes, if pressure will rise up suspected system is leakage. Do find the leakage spot with using soap, If leakage found out do assist to rectify or minimize the leakage.
4. If the pressure in manifold gauge is remain in vacuum state the system is no leakage. Do assist to top up gas as per unit nameplate.
5. Please revert the outcome.

DCHS

Unit CGMU6506806  Model:LXE10E-149 with setpoint -23C ,

1. Thank for the efforts in repairing the unit.

2. Observed DCHS low causing EEV opening abnormal.

3. Do ensure DCHS temp sensor properly secure at discharging pipe or properly inserted to the tube and reset unit power and observe EEV opening and temperature improvement. DCHS temp should between +70 to +90C

4. Do revert outcome.

BRAZING

Unit CGMU4947119 Model: LXE10E-A15J and with Set Temp of -18.0C.
1. Thanks for the efforts in repairing the unit.
2. Do replace filter drier, if possible, do replace also the LSV body if brazing is possible on board, no any DG cargo near, be careful when replacing the LSV body, if overheated, leakage of LSV head may occur.
3. Do assist to check EEV body for frosting, if yes, do find attached for EEV servicing.
4. Do revert outcome.

60% HUMIDITY

Kindly assist to advise and clarify the correct Humidity setting for subject unit.
As per on the manifest Humidity setting is 50%, but the DAIKIN unit model LXE10E Humidity setting range is only with the minimum of 60% and the maximum of 95%. Please be informed that humidity setting cannot be set below 60% or above 95%. Please advise if the 60% setting can be consider, since the required setting on manifest is not possible to apply for this type of unit.

EEV

Container ID: KKFU6578930, Model: LXE10E-A9C & SP: -1.7C.

1. Thanks for feedback.

2. Is the EEV body still cover by ice? If yes, try to remove the EEV coil then operate the unit without EEV coil.

3. If EEV body not frosted after removing the EEV coil which mean EEV body not chocked. Do refit EEV coil by hand tight only if ice starting build-up on EEV body do re-adjust/loosen the EEV coil until ice formation gone.

4. Do revert outcome with latest parameter values.

Unit CGMU6537350 Model:LXe10E-136 with setpoint 0C ,

1. Thanks for feedback.

2. Do assist to release or recover all gas, replace new filter drier and fully vacuum all 5 service ports, first: vacuum the service port #4 (after the check valve) and port #2 (discharge service port), second: transfer to port #1( suction port) and service port #5 lastly transfer high side hose to port number #3, transferring of manifold hose every 30 minutes. Do ensure while system vacuuming, no open service ports, it should be covered by port caps to avoid entry of air. Do find attached photo for service port location and its numbering.

3. After evacuating, do observe the manifold gauge within 20 minutes, if pressure will rise up suspected system has a leakage. Do find the leakage spot with using soap, If leakage found out do assist to rectify or minimize the leakage.

4. If the pressure in manifold gauge that was connected to port #1 and port #3 was remain in vacuum state which means the system has no leakage. Do assist to top up the exact amount of liquid refrigerant at port #3, the exact amount of refrigerant to be charge on the unit is based on the unit nameplate. Use the weighing scale to measure the exact amount that charged on the unit.

5. Do revert outcome.